Captions are under each screenshot. Spellcheck was unavailable in WordPad. I tried very hard to be complete, but used some editorial discretion when things were very repetitive. I have been paid to do technical documents professionally so I know it could be better but for this WordPad was good enough. If you are using the RTF version, WordPad will now show a warning about the trustworthiness of the file.  Not sure why.  Win10 also refuses to associate RTF with WordPad to the small extent I tried.  WordPad will also open the OpenOffice file format that it was also saved in but working version was saved in .RTF.  The "details" for non-obvious items might be under the first module screenshot or the last.  I was not particularly consistent.  I aspired to fix but probably won't.

Manually transcribed from ForScan each entry that was not an obvious binary choice. A few apparent binary had a 3rd value 0 choice NONE or similar.

Sample vehicle is a Ford Maverick Model Year 2024 Hybrid with 2.5L engine and XLT trim and no apparent enhancements. For most runs, the vehicle was in the RUN position, not just ACC ON.  It took so long and I did not want to wear down the battery.  The gasoline engine fired up at points. The only change from shipped was 4-door unlock done via the normal cluster menu.

BdyCM - 1

Various Step Sizing and Time - NO LIMITS

Ambient Lighting Color - Choice of 16

BdyCM-2

BdyCM-3

Axle Ratio - NO LIMITS

BMS Configuration 0-NA, 1-LIN, 2-PMW

Battery Technology 0-Flooded, 1-AGM

Body Mode 0-Build, 1,Normal, 2-Transport

Collision Mitigation - 1-NONE, 2 Collision mitigation by branking GEN 1, 3-GEN 2, 6-GEN 3

Courtesy Lighting 0-Bulb, 1-LED

Cruisee Control 1-Without; 2 Cruise Control; 3 Adaptive CC, 4- Adaptive + Stop&Go; 6-Adaptive, Brake Limit 2; 
7-Adapative+S&Gc BL2; 8-Adaptivec, ACC to CC toggling; 9-Adaptive+S&G, ACC to CC toggling;
16-Adaptive+BL2, ACCC to CC; 17 Adaaptive+S&G, BL2, ACC to CC

BdyCM-4

BdyCM-5

BdyCM-6

BdyCM-7

BdyCM-8

BdyCM-9

BdyCM-10

BdyCM-11

ABS-1 - I am guessing these might be READ-ONLY from what is installed.

ABS Evacuate and Fill: 0-No Fill, 1=Production line, 2 Service, 3 Production Repair Fill

Body Style 4-5 Door SUV, 6-Crew Cab Truck

Calipers: 1-CI135, 2-CI185 (Something close to that.  My eyes aren't great and my screen is cracked.)

Collision Mitigation: 2-Gen 1, 5=Gen 2.

Cruise Control: 1-None, 2-Cruise Control, 3-Adaptive Cruise Control, 4-Adaptive+Stop&Go.

Drivetrain: 1-FWD, 2-AWD

ECU Configuration Firmware: 1- Incorrect or not configured (This is mine set), 2-Maverick Basic NZ6C-4C602*, 5-Maverick DECAL-FX4-NZ6C4602-RA*, 6-Maverick FX4- OFF ROAD NZ6-14602-SA*, 253-Bronco Sport LX6C-14C602-NA*

Ford Stability Control 2-No Roll Rate, 3-With Roll Raat

Gearbox 7-8f35 (E-Shifter), 10-H4F45 FHEV/PHEV

SDM Driveline: 0-Disabled/FWD, 1-TCCM, 2-DLCM/AWD

SDM Switch Sourcw ECU: 0-ATCM/SDM Offroad, 1-TCM/PCM 2-HVAC/SDM Onroad, 3-SCCM

SDM Switch Type: 0-Rotary/Offroad, 1-Push Button, 2=Rocker, 3-Rotary with OK, 4-None, 5-Rocker & Direct.

Suspension Type: 1-Standard, 8-Perfomance(Active), 9-Rough Road, 10-SPECIAL.

Tires: 1-17in 102H A/S, 3-18in 100H A/S, 10-225/65 R17 A/T, 11-235/65R17 A/T

Vehicle Series: 1-Base, 2-Offroad.

ABS-2

ABS-3

HVAC-1

AC Compressor: 0-Fixed, 1-Internally Variable, 2=Externally Variable, 3-Electric

AUTO Seat Function Type: 0-Not Supported, 1-Supported No Button, 2- Supported dedicated button

AUTO Seat Function Type: 0-Not Supported, 1-Supported No Button, 2-Supported with Button (WDTM-IDK)

Automatic Transmission Warm-up Type: 0-Climate Thermal Load, 1-Engine Coolant and Outside Air.

Aux HVAC Unit Type: 0-Independent Aux HVAC, 1-Console Unit Connected to Front HVAC

Climate Control Type: 0-Automatic, 1-Manual

Display Type: 0-None, 1-Touchscreen, 2-Centerstack

Engine Type: 0-Gasoline, 1-Hybrid, 2-Diesel, 3-Plugin Hybrid

Engine Type Based Strategy: 0-Gasoline, 1-FHEV, 2-Diesel, 3-PHEV, 4-BEV

Front Control Zone: 0-Single, 1-Dual

Front In-car Temperature Sensor: 0-Not Present, 1-IP Mounted, 2-Windshield Mounted.

Front Left Seat High ^C: INTEGER

Front Left Seat Low ^C: INTEGER

Front Left Seat Medium ^C: INTEGER

PDM/DDM Modules: 0-Not Present, 1-Equipped

PTC Heater Type: 0-None Present, 1-Two Relay, 2-Three Relay, 3-PWM

Front PTC Wattage: Choice of 16 options 0 Watt-2500 Watt.

Front Right Seat High ^C: INTEGER

Front Right Seat Low ^C: INTEGER

Front Right Seat Medium ^C: INTEGER

HVAC Core: 0-Heater and Evaporator, 1-Heater Only

Human Interface State Matric: 0-North America, 1-Europe, 2-Asia

Humidity Sensor Type: 0-Not Present, 1-IP Mounted, 2-Windshield Mounted

Prepare for AUTO A/C Evaluation: 0-Manual, 1-Discharge sensor base and auto check.

Rear Blower Knob: 0-Fixed Detents, 1-Continuous

Rear Climate Zones: 0-Single, 1-Dual

Rear Control Zone: 0-Single, 1-Dual, 2-Synced Dual

Rear In-car Temperature Sensor Type: 0-Not Present, 1-IP Mounted, 2-Windshield Mounted

Rear PTC Heater Type: 0-None Present, 1-Two Relay, 2-Three Relay, 3-PWM

Rear PTC Wattage: Choice of 16 options 0 Watt-2500 Watt.

Region: 0-North America, 1-Europe, 2-Asia, 3-South America

Seat Fabric: 0-Cloth, 1-Leather

Second Row Control Type: 0-None, 1-Manual, 2-Automatic, 3-3rd Party

Sun Load Sensor Type: 0-Disabled, 1-Single, 2-Dual

Third Row Control Type: 0-None, 1-Manual, 2-Automatic, 3-3rd Party

Third Row In-car Temperature Sensor: 0-Not Present, 1-IP Mouunted, 2-Windshield Mounted

Vehicle Driver Location: 0-Left Hand Drive, 1-Right Hand Drive

Wide Open Throttle Purge Type: 0-None, 1-Pedal Based Purge, 2-Enrichment Purge

HVAC-2

HVAC-3

HVAC-4

HVAC-5

HVAC-6

PSCM

ABS Type: 0-None, 1-ABS Only, 2-ABS+ESC

Engine Torque Class: 0-Lowest, 1-Low, 2-Medium, 3-High, 4-Highest

Software End Stops: 0-Disabled, 1-Incorrect or Not Configured, 255-Enabled.

ACM-1

ACM-2

Front Camera: 0-Not Configured, 1-Not Present, 2- Present(no split), 3-Present (split view)

Leading zero for clock: 0-Not configured, 1-With leading zero, 2-Without leading zero

Media Indexing: 0-Not configured, 1-Enabled, 2-Disabled

Media Sorting: 0-Standard, 1-FSAO

Output Channel Mode: 0-Not Configured, 1-2 Channels, 2-4 Channels

Parking Aid Configuration: 0-Manual transmission, 1-Autmatic transmission.

Parking Aid Feature Set: 0-Not Configured, -1- No PDC/PSM/SAPP, 2-Rear PMA Only/No SAPP, 3-Front and Rear PAM/No SAPP, 4-Rear PAM/SAPP, 5-Front and rear PAM/SAPP, 6-12 channel Park Aid (SAPP, PPA, POA + Flank Guard), 7-POA+SAPP, 8-SAPP, 11-Full Active Park Assist

Police Video On Demand: 0-Not Configured, 1-Disabled, 2-Enabled. (Generally I dont document all the ENABLE/DISABLE but this one looked interesting)

Rear View Camera: 0-Not configured, 1-Not present, 2-Present non-NA layout, 3-Present NA layout, 4 Present non-NA layoutt with Split View option.

SDARS Data Services: 0-Not Configured, 1-Data Service (US Data Tier), 2-No Data Services

SDARS Region: 0-Not Configured, 1-USA, 2-Canada, 3-No Content

Speaker 1 LF: 0-Non Tweeter only, 1-Tweeter only, 2-Woofer & Tweeter, 3-No speaker test

Speaker 2 RF: 0-Non Tweeter only, 1-Tweeter only, 2-Woofer & Tweeter, 3-No speaker test

Speaker 3 RR: 0-Non Tweeter only, 1-Tweeter only, 2-Woofer & Tweeter, 3-No speaker test

Speaker 1 LR: 0-Non Tweeter only, 1-Tweeter only, 2-Woofer & Tweeter, 3-No speaker test

Speaker configuraton: 0-Not Configured, 1-2 speakers, 2-6 speakers, 3-8 speakers, 4-4 speakers, 5-10 speakers, 6-12 speakers, 7-Less speakers, 8-4 speakers, 9-9 speakers, 10-9 Radio Speakers Low, 11-7 speakers, 16-11 High Speakers (High Speaker Set), 17-11 High Speakers (9 Channel Audiophile), 18-13 High Speakers (8 channel Audiophile LMV), 19-4 front speaker & 2 coaxial rear speaker, 20-10 High Series speakers with Subwoofer, 21-9 Speaker ST, 255 - Any

System Name: 0-Not Configured, 1-Ford Audio, 2-Ford Sync

Time Format: 0-Not Configured, 1-12h, 2-24h

Time Zone: Select from 40 choices based on UTC format including 30 min offset options

Timing: 0-Not Configured, 1-10 min, 2-25s, 3-Disabled

Time Knob Accelerattion: 0-Not Configured, 1-OFF, 2-ON, 3-Disabled

Tuner Frequency Band and Step: 0-Not Configured/Default (EU/GCC), 1-Europe/GCC, 2-North America, 4-Japan, 6-All Other, 9-South America

USB: 0-Not configured, 1-No USB, 2-USB 1 Only, 5-USB 1+MCM

Vehicle Settngs: 0-Not Configured, 1-Disabled, 2-Enable

Vehicle Driver Location: 0-Not Configured, 1-Left, 2-Right, 255-ANY

Vehicle Playback Restriction: 0-Not configured, 1-Disabled, 2-Enable with restrictions 3-Enable with speed restriction, 4-Enable with reverse and speed restrictions, 5-Enable with park brake and speed restrictions, 6-Enable with ignition NOT in RUN position

Video Playback Speed Lock: 14 choices from 0 km/h to 23 km/h

SCCM

Heated Steering: Range of 16 choices from 24^C to 15^C

Steering Wheel Switch LEFT: Range of 32 choices from some sort of part or model codes

Steering Wheel Switch RIGHT: Range of 16 choices from some sort of part or model codes

IPC Odometer Write Warning

IPC-1

IPC-2

IPC-3

IPC-4

IPC-5

IPC-6

IPC-7

IPC-8

IPC-9

IPC-10

IPC-11

IPC-12

IPC-13

IPC-14

IPC-15

IPC-16

4x4/AWD Source Module: 0-Not Used, 1-TCCM, 0-AWD (I don't understand reuse), 3-PCM

Auto High Beam Telltale Source: 0-Disabled, 1-IPMA, 2-SCCM

Average Fuel Economy Display Type: 0-L/100 km, 1-km/L

Back Cursor Button: 0-Not Present, 1-Present

Boost Vacuum Range: 0-Range 1 thru 3-Range 4

Brake Telltale Symbol: 0-USA, 1-All Others

Brake Type: 0-Hydraulic, 1-Air Brakes

Brand: 0-Ford, 1-Lincoln

Chimes Primary Source: 0-Instrument Cluster Only, 2-Audio Speakers Only

Chimes - Vehicle running with door open: 0-Disable, 1-Automatic Transmission, 2-Manual Transmission

Cluster Display Settings Menu: 0-Cluster, 1-Center Stack

Country Code: Free Text Unknown Size

DTE Empty Value: INTEGER (presumably Distance to Empty)

DTE: 0-Distance based algorithm, 1-Fuel level (volume) based algorithm

DTE Rate of Change: 0-Descending, 1-Ascending/Descending

DTE Values: 0-Normal, 1-Decreased

Date Time Strategy: 0-Julian Day, 1-Day of Month, 2-Other

Display Temperature Default Unit: 0-F, 1-C

Display Unit Selection Function: 0-Imperial, 1-Metric

Door Content: 0-Door/Rear Cargo Doors, 1-Liftgate/Liftglass, 2-Liftgate, 3-Trunk

Drive Train Type: 0-2WD, 1-4WD, 2-AWD

Engine Brake Guage Display: 0-Not present, 1-Present

Engine Fuel Type: 0-Unused, 1-Gasoline, 2-Diesel, 3-Others

Flex Fuel: 0-No, 1-Yes

Ford Performance Variant: 0-Not Present, 1-Present

Forward Collision Warning: 0-None, 1-Forward Collision Warning (FCW), 2-FCW+Following Distance Alert (FDA), 3-Unused

Fuel Economy Method: 0-Total Distance, 1-3200km sawtooth

Fuel Tank Arrow Direction: 0-Left, 1-Right

Fuel Tank Size, Gallons: INTEGER

Fuel Tank Size, Liters: INTEGER

Fuel Level Source: 0-IPC, 1-PCM

GPS Display: 0=Not Present, 1-Present

Gallon Display Type: 0-US, 1-UK

Gear Ratio: 0-6R, 1-10R

Global Clock display format: 0-12H, 1-24H

Heads Up Display Button: 0-Not present, 1-Present

Heads Up Display (HUD): 0-No HUD, 1-Foward Collision Warning HUD, 2-Advanced HUD, 3-TFT HUD

High Beam Telltale Source: 0-BCM, 1-HCM, 2-SCCM

Home Button: 0-Not present, 1-Present

Instant Fuel Economy Display: 0-Bar, 1-Numeric

Intelligentf Access OR KeyFree Text: 0-IA, 1-KF

Left Cursor Button: 0-Not Present, 1-Present

Memory Switch Type: 0-Physical switches, 1-Touchscreen switches on center stack

Menu - Auto High Beam: 0-Disabled, 1-Auto high beam, 2-Glare free high beam, 3-Adaptive Driving beam

Menu button: 0-Not Present, 1-Present

Message Center Display Interface Type: 0-S-Family, 1-Display with Vision wheel, 2-Display with QAM buttton, 3-Mustang display with QAM button, 4-Steering Horizon controlled

Minor Gauge Units: 0-Alphabet, 1-Numeric

Multiple Chime Signal: 0-Single, 1-Dual

MyKey 911 Text: 0-911, 1-Emergency

MyKey Speed Limit Market: 0-Disabled, 1-USA, 2-Canada/Mexico/South America, 3-UK, 4-All other

Navigation Button: 0-Not Present, 1-Present

Navigation Speed  Sign: 0-North America, 1-Rest of World

Numbe of Fuel Sensors: 0-1 sender, 1-2 senders

One/Two stage Unlocking Passenger/Commercial: 0-Passenger, 1-Commercial

Outside Air Temperature Source: 0-HVAC, 1-Front Controls Interface

Perimeter Alarm Text: 0-All Sensors, 1-Motion Sensors

Phone Button: 0-Not Present, 1-Present

Power Liftgate Chime: 0-Type 1, 1-Type 2

Power Sliding Door: 0-None, 1-Left, 2-Right, 3-Both

Powertrain Hybrid: 0-Non-Hybrid, 1-Battery Electric Vehicle, 2-Full Hybrid, 3-Plugin Hybrid

Rear Belt Monitor Seating Layout: 13 choices of various seat counts

Rear Belt Monitor Strategy: 0-Non-European, 1-European

Removable Doors: 0-Not Present, 1-Present

Right Cursor Button: 0-Not Present, 1-Present

Seat Row 1 Bench: 0-Not Present, 1-Present

Seat Row 2 Bench: 0-Not Present, 1-Present

Seat Row 3 Bench: 0-Not Present, 1-Present

Seatbelt Configuration - Row 1-5, Col 1-5 (Repetitive):0-Seat Not Present, 1- Fixed seat present w/ Belt Minder, 2-Fixed seat present with belt monitor, 3-Removable seat with Belt Minder, 4-Removable seat w/ belt monitor, 5-Fixed seat without buckle sensor.

Seatbelt Warning Market: 0-USA, 1-Europe

Select Shift Type Mode: 0-"S", 1-"M"

Send Button: 0-Not Present, 1-Present

Settings Menu Location: 0-Cluster, 1-Center Stack

Shift-By-Wire eShifter Type: 0-Push Button, 1-Rotary Dial

Speedometer Police Cal: 0-Non-Police, 1-Police

Speedometer Secondary Scale: 0-Not Present, 1-Present

Speedometer calibration: 0-Not Present, 1-Present

Splash Screen Type: Range of 8 choices by number, no clarifications.

TCCM Neutral Tow Capable: 0-No, 1-Yes

TPMS Default Units: 0-psi, 1-kPa, 2-Bar, 3-Bar

TPMS Hard Button Reset: 0-Disabled, 1-Enabled

TPMS by Location Units: 0-None, 1-psi, 2-kPa, 3-Bar

Tachometer Redline: 0-None, 1-Range 1, 2-Range 2, 3-Range 3

Temperature Curve: 0-Standard, 1-Hot

Total Engine Run Time, s: INTEGER (mine says 0)

Total Idle Run Time, s:INTEGER (mine says 0)

Total Odometer Delta Distance Added, km: decimal/float (mine says 0.0)

Total Run Time with PTO, s:INTEGER (mine says 0)

Trailer Measurement: 0-ABCD, 1-ABC, 2-A, 3-None

Transfer Case Type: 0-Part Time, 1-Full Time

Transmission Temperature Curve: 0-Table A (10R80, 8F24, 8F57, 9F34, 9F40, 8F35), 1-Table B (10R100, 10R140, 6F50, 6F55, 6F35, 6F15), 2-Table C (6R80, 6R140, 6R100, HF35, HF45)

Vehicle Settings Button: 0-Not Present, 1-Present

Vehicle Driver Location: 0-Left Hand Drive, 1-Right Hand Drive

View Button: 0-Not Present, 1-Present

Virtual Speedometer Style: 0-Style #1, 1-Style #2, 2-Style #3, 3-Style #4

Warning Display Location: 0-Cluster, 1-Center Stack

IPMA-1

IPMA-2

IPMA-3

IPMA-4

Country: List of 15 choices.  Includes USA.  Not sure what significance of any are.  1-Rest of the world.

Customer Preferences - ACC Gap Setting Personal 1-4  : 0-Not used, 1-Gap #1, 2-Gap #2, 3-Gap #3, 4-Gap #4, 5-Gap #5

Customer Preferences - ACC Gap Setting Vehicle: 0-Not used, 1-Gap #1, 2-Gap #2, 3-Gap #3, 4-Gap #4, 5-Gap #5

Customer Preferences - FCW Sensitivity - Personal 1-4: 0-Not Used, 1-Short/Low, 2-Normal, 3-Long/High

Customer Preferences - FCW Sensitivity - Vehicle: 0-Not used, 1-Gap #1, 2-Gap #2, 3-Gap #3, 4-Gap #4, 5-Gap #5

Customer Preferences - Lane Keep Alert Intensity Personal 1-4: 0-Off, 1-Low, 2-Medium, 3-High

Customer Preferences - Lane Keep Alert Vehicle: 0-Off, 1-Low, 2-Medium, 3-High

Customer Preferences - Lane Keep Alert Sensitivity Personal 1-4: 0-Normal, 1-Level 2

Customer Preferences - Lane Keep Alert Sensitivity Vehicle: 0-Normal, 1-Level 2

Customer Preferences - Lane Keep Settings Personal 1-4: 0-Not Used, 1-LK Alert, 2-LK Aid, 3 LK Alert + LK Aid

Customer Preferences - Lane Keep Settings Vehicle: 0-Not Used, 1-LK Alert, 2-LK Aid, 3 LK Alert + LK Aid

Engine Type: Range of 14 choices for various size/Turbo or Not and type including Hybrid types.

Forward Collision Warning on HUD: 0-None, 1-No Head-Up display, 2-HUD, 3-Advanced HUD

Headlamp Type: 0-None, 1-Halogen, 2-Xenon, 3-LED

High Beam Configuration: 0-None, 1-Disabled, 2-Auto High Beam Control, 3-Glare free High Beam/Adaptive Driving Beam

Lane Assist: 0-None, 1-Disabled, 2-Lane Departure Warning (LDW), 3-LDW+LKA

Lane Assist Type: 0-None, 1-Haptic, 2-Power, 3-Adaptive

Lane Assist Switch: 0-None, 1-Direct, 2-Network, 3-No Switch

Pre-Collision Assist Type: 0-None, 1-Disabled, 2-Radar Fusion, 3-Camera Only

Region: 0-None, 1-Europe, 2-North America, 3-South Africa, 4-China, 5-Asia Pacific, 6-Africa, 7-Gulf Cooperation Council (GCC), 8-Australia / New Zealand

Speed Limit Sign Unit: 0-Unused, 1-km/h, 2 mph

Traffic Jam Assist: 0-None, 1-Disabled, 2-Enabled

Traffic Sign Recognition (TSR): 0-None, 1-Disabled, 2-SLOIF, 3-Speed Limit Information Function (SLIF)

Traffic Sign Recognition Mode: 0-None, 1-Disabled, 2-Camera only, 3-Fusion (map+camera)

Vehicle Type: Bunch of choices. No obvious order or count.

Vehicle Driver Location: 0-None, 1=Right Hand drive, 2-Left Hand drive

Wheel arch height, left front, mm: INTEGER

Wheel arch height, right front, mm: INTEGER

Windshield Heater: 0-None, 1-Direct, 2-Network

Windshield Type: 0-None, 1-Basic, 2-Acoustic, 3-IR Coated

Wrog Way Alert: 0-None, 1-Disabled, 2-Enabled