FLAT TOWING THE MAVERICK

Switching Neutral Tow On

1. Switch your vehicle on in accessory

mode.

2. Press the menu button on the steering

wheel to enter the information display

main menu.

3. Select Settings.

4. Select Neutral Tow.

5. Follow the instructions on the

information display.

6. Fully press the brake pedal.

7. Shift into neutral (N).

8. Switch the ignition off.  Leave key in ignition.

9. Turn ON motorhome lights or park lights.

Note: Switching neutral tow on requires

battery power.

Note: If the parking brake is applied, a

message appears in the information display

Note: If your vehicle has an ignition key, you

cannot remove the key from the ignition

when the transmission is in neutral (N) and

your vehicle is off.

Note: Start the engine and allow it to run

for a few minutes at the beginning of each

day, and every six hours or fewer. With the

engine running and your foot on the brake,

shift into drive (D) and then into reverse (R)

before shifting back into neutral (N). Before

continuing to tow, switch neutral tow on.

Check towbar hookup and lights.