2023 Ford Maverick Hybrid 2.5L
Maintenance Logbook

VIN #: 3FTTW8E38[____ ____]

Fluid/consumables guide

| OIL | Model |
| --- | --- |
| Filter | Motorcraft FL-910-S |
| type | WSS-M2C962-A1 (Motorcraft 0W-20) |
| capacity | 5.7 quarts (includes oil filter) |
| Drain Plug ¦ lower cover | 15mm & 22 ft-lbs ¦ TP27 (torx plus, preferred) or T30 lower splash guard bolts |
| Reset oil life (OCI) | Settings → Oil life reset → Hold OK to reset → Press/hold shows success 100% |

| Coolant | Model |
| --- | --- |
| type | Motorcraft Yellow Prediluted Antifreeze ¦ WSS-M97B57-A2 |
| capacity | Low temp system: 4.8 qt ¦ High Temp system (engine): 11.7 qt ¦ Total: 16.5 quarts |

| transmission fluid | Model |
| --- | --- |
| type | Motorcraft Mercon ULV ¦ XT-12-QULV (1 Quart) / XT-12-5QULV (5 Qt Jug) |
| Capacity | 4.8 quarts |

| Misc | Model |
| --- | --- |
| AIR FILTER (cabin) | FP-100-A (standard) or FPP-89 (premium) |
| AIR FILTER (engine) | FA-1948 |
| Battery | BXT-99RT4-A ¦ Group 99R / T4 (factory) or Group 140R / LN1 |
| Brake fluid | WSS-M6C65-A2 (Motorcraft DOT 4 LV High Performance) |
| lug nuts | 21mm / 148 ft-lbs |
| Spark plugs | SP-530 |
| Tire size | 225/65R17 |
| Windshield wipers | WW-2249 (driver) WW-2054 (passenger) ¦ 22” Driver / 21” Pax Pinch Tab design |

Normal scheduled maintenance:

| Maintenance Schedule | due |
| --- | --- |
| oil | Per OCI or annually whichever comes first |
| cabin air filter | 20,000 miles |
| engine air filter | 30,000 miles |
| Transmission fluid | 30,000* miles (150,000 per Ford) |
| Engine Coolant | 50,000* miles (200,000 then every 5 years/100,000 miles per Ford) |
| Spark plugs | 100,000 miles |
| Brake fluid | 3 years |

Tire rotation pattern:

(Front)

*Adjusted for personal preference

| Date | Mileage | Activity | Location |
| --- | --- | --- | --- |

| Date | Mileage | Activity | Location |
| --- | --- | --- | --- |

| 6 months Current date: ______________ Current miles: ______________ |  |  | OCI reminder / 1 year Current date: ______________ Current miles: ______________ |  |
| --- | --- | --- | --- | --- |
| Inspection item | I = inspect R = replace |  | Inspection item | I = inspect R = replace |
| Engine Oil level | I □ |  | Change oil / replace filter | R □ |
| Interior/Exterior lights | I □ |  | Rotate tires | R □ |
| Tire tread, condition, and pressure including spare (35 psi) | I □ |  | Transmission fluid inspection | I □ |
| Washer fluid level, spray, and wiper blades | I □ |  | Inspect brake pads, rotors, hoses, parking brakes | I □ |
| 12v battery terminals for corrosion | I □ |  | Coolant levels | I □ |
| Body/door drain holes | I □ |  | Exhaust and heat shields | I □ |
| Coolant levels | I □ |  | CV Axles | I □ |
| Door weather stripping, lubricate if required | I □ |  | The steering linkage, ball joints, suspension, tire-rod ends | I □ |
| Lubricate hinges/latches if required | I □ |  |  |  |
| Parking brake for operation | I □ |  |  |  |
| Seatbelts and latches | I □ |  |  |  |
| CEL and associated warnings | I □ |  |  |  |
|  |  |  | Multi – Point Inspection |  |
| 6 month notes: |  |  | All items from 6-month inspection | I □ |
|  |  |  | 12v Battery performance / corrosion | I □ |
|  |  |  | Engine & cabin air filter | I □ |
|  |  |  | Exhaust system | I □ |
|  |  |  | Oil and fluid leaks | I □ |
|  |  |  | Half-shaft dust boots | I □ |
|  |  |  | Hazard warning system operation | I □ |
|  |  |  | Horn operation | I □ |
|  |  |  | Radiator, cooler, heater and air conditioning hoses | I □ |
| OCI / 1 year notes: |  |  | Suspension components for leaks or damage | I □ |
|  |  |  | Steering and linkage | I □ |
|  |  |  | Windshield for cracks, chips or pits | I □ |
|  |  |  | Major maintenance Tasks completed |  |
| 6 months Current date: ______________ Current miles: ______________ |  |  | OCI reminder / 1 year Current date: ______________ Current miles: ______________ |  |
| Inspection item | I = inspect R = replace |  | Inspection item | I = inspect R = replace |
| Engine Oil level | I □ |  | Change oil / replace filter | R □ |
| Interior/Exterior lights | I □ |  | Rotate tires | R □ |
| Tire tread, condition, and pressure including spare (35 psi) | I □ |  | Transmission fluid inspection | I □ |
| Washer fluid level, spray, and wiper blades | I □ |  | Inspect brake pads, rotors, hoses, parking brakes | I □ |
| 12v battery terminals for corrosion | I □ |  | Coolant levels | I □ |
| Body/door drain holes | I □ |  | Exhaust and heat shields | I □ |
| Coolant levels | I □ |  | CV Axles | I □ |
| Door weather stripping, lubricate if required | I □ |  | The steering linkage, ball joints, suspension, tire-rod ends | I □ |
| Lubricate hinges/latches if required | I □ |  |  |  |
| Parking brake for operation | I □ |  |  |  |
| Seatbelts and latches | I □ |  |  |  |
| CEL and associated warnings | I □ |  |  |  |
|  |  |  | Multi – Point Inspection |  |
| 6 month notes: |  |  | All items from 6-month inspection | I □ |
|  |  |  | 12v Battery performance / corrosion | I □ |
|  |  |  | Engine & cabin air filter | I □ |
|  |  |  | Exhaust system | I □ |
|  |  |  | Oil and fluid leaks | I □ |
|  |  |  | Half-shaft dust boots | I □ |
|  |  |  | Hazard warning system operation | I □ |
|  |  |  | Horn operation | I □ |
|  |  |  | Radiator, cooler, heater and air conditioning hoses | I □ |
| OCI / 1 year notes: |  |  | Suspension components for leaks or damage | I □ |
|  |  |  | Steering and linkage | I □ |
|  |  |  | Windshield for cracks, chips or pits | I □ |
|  |  |  | Major maintenance Tasks completed |  |

On the Road Maintenance

| Date | Mileage | Activity | Location |
| --- | --- | --- | --- |

Fluid/consumables guide

| OIL | Model |
| --- | --- |
| Filter | Motorcraft FL-910-S |
| type | WSS-M2C962-A1 (Motorcraft 0W-20) |
| capacity | 5.7 quarts (includes oil filter) |
| Drain Plug ¦ lower cover | 15mm & 22 ft-lbs ¦ TP27 (torx plus, preferred) or T30 lower splash guard bolts |
| Reset oil life (OCI) | Settings → Oil life reset → Hold OK to reset → Press/hold shows success 100% |

| Coolant | Model |
| --- | --- |
| type | Motorcraft Yellow Prediluted Antifreeze ¦ WSS-M97B57-A2 |
| capacity | Low temp system: 4.8 qt ¦ High Temp system (engine): 11.7 qt ¦ Total: 16.5 quarts |

| transmission fluid | Model |
| --- | --- |
| type | Motorcraft Mercon ULV ¦ XT-12-QULV (1 Quart) / XT-12-5QULV (5 Qt Jug) |
| Capacity | 4.8 quarts |

| Misc | Model |
| --- | --- |
| AIR FILTER (cabin) | FP-100-A (standard) or FPP-89 (premium) |
| AIR FILTER (engine) | FA-1948 |
| Battery | BXT-99RT4-A ¦ Group 99R / T4 (factory) or Group 140R / LN1 |
| Brake fluid | WSS-M6C65-A2 (Motorcraft DOT 4 LV High Performance) |
| lug nuts | 21mm / 148 ft-lbs |
| Spark plugs | SP-530 |
| Tire size | 225/65R17 |
| Windshield wipers | WW-2249 (driver) WW-2054 (passenger) ¦ 22” Driver / 21” Pax Pinch Tab design |

Normal scheduled maintenance:

| Maintenance Schedule | due |
| --- | --- |
| oil | Per OCI or annually whichever comes first |
| cabin air filter | 20,000 miles |
| engine air filter | 30,000 miles |
| Transmission fluid | 30,000* miles (150,000 per Ford) |
| Engine Coolant | 50,000* miles (200,000 then every 5 years/100,000 miles per Ford) |
| Spark plugs | 100,000 miles |
| Brake fluid | 3 years |

Tire rotation pattern:

(Front)

*Adjusted for personal preference

Rest of pages are blanks to copy/paste/print as needed

| Date | Mileage | Activity | Location |
| --- | --- | --- | --- |

Table width 7.5”, Cell widths 1”, 1”, x”, 1.5” (reset as needed)

| 6 months Current date: ______________ Current miles: ______________ |  |  | OCI reminder / 1 year Current date: ______________ Current miles: ______________ |  |
| --- | --- | --- | --- | --- |
| Inspection item | I = inspect R = replace |  | Inspection item | I = inspect R = replace |
| Engine Oil level | I □ |  | Change oil / replace filter | R □ |
| Interior/Exterior lights | I □ |  | Rotate tires | R □ |
| Tire tread, condition, and pressure including spare (35 psi) | I □ |  | Transmission fluid inspection | I □ |
| Washer fluid level, spray, and wiper blades | I □ |  | Inspect brake pads, rotors, hoses, parking brakes | I □ |
| 12v battery terminals for corrosion | I □ |  | Coolant levels | I □ |
| Body/door drain holes | I □ |  | Exhaust and heat shields | I □ |
| Coolant levels | I □ |  | CV Axles | I □ |
| Door weather stripping, lubricate if required | I □ |  | The steering linkage, ball joints, suspension, tire-rod ends | I □ |
| Lubricate hinges/latches if required | I □ |  |  |  |
| Parking brake for operation | I □ |  |  |  |
| Seatbelts and latches | I □ |  |  |  |
| CEL and associated warnings | I □ |  |  |  |
|  |  |  | Multi – Point Inspection |  |
| 6 month notes: |  |  | All items from 6-month inspection | I □ |
|  |  |  | 12v Battery performance / corrosion | I □ |
|  |  |  | Engine & cabin air filter | I □ |
|  |  |  | Exhaust system | I □ |
|  |  |  | Oil and fluid leaks | I □ |
|  |  |  | Half-shaft dust boots | I □ |
|  |  |  | Hazard warning system operation | I □ |
|  |  |  | Horn operation | I □ |
|  |  |  | Radiator, cooler, heater and air conditioning hoses | I □ |
| OCI / 1 year notes: |  |  | Suspension components for leaks or damage | I □ |
|  |  |  | Steering and linkage | I □ |
|  |  |  | Windshield for cracks, chips or pits | I □ |
|  |  |  | Major maintenance Tasks completed |  |

| FORSCAN EDITS DO ALL IN SEQUENTIAL ORDER |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Edit | Module | Parameter | Hex 1 | Hex 2 | Hex 3 | Notes |
| Lane Keep System retrofit w/ stalk | IPC | 720-01-02 | x2xx |  |  | 2 enables Driver alert system (Factory digit for non co-pilot is a 0) |
|  |  | 720-02-01 | xxEx |  |  | E enables Lane assist, changes later to 6 if key in ignition OFF selected |
|  |  | 720-09-01 |  |  | x9-- | Enables Lane assist Haptic selection menu (LAHI) |
|  | PSCM | 730-02-02 | FFFF |  |  | Enables Lane keep assist and Lane departure warning |
|  | IPMA | 706-01-01 |  | x9Cx | Bx-- | Activates functionality in the image processing module. Set B to A later w/ CCM radar add |
|  | SCCM | 724-02-01 | xx58 | 6xxx |  | Steering wheel buttons w/ cruise control. 586 changes to 906 w/ Lane Centering button add |
| Adaptive Cruise Control w/ lane centering retrofit LINK | BdyCM | 726-04-05 |  |  | 90xx | Adds adaptive cruise control and CC mitigation by braking, Gen 1 |
|  | ABS | 760-03-02 | x4xx |  |  | Adds adaptive speed control Stop/Go |
|  | SCCM | 724-02-01 | xx90 |  |  | Adds lane centering button functionality (update from Lane Keep Assist retrofit done earlier) |
|  | PSCM | 730-02-02 | FFFF | FFFF |  | Power steering control module, adds Lane Keep Assist from Lane Keep system retrofit |
|  |  | 730-02-03 | FFxx |  |  | Enables Evasive Steering Assist |
|  | IPC | 720-01-01 | xFxx |  |  | Enables Adaptive Cruise Control menus |
|  |  | 720-01-02 |  | 8xxx |  | Adds forward collision warning |
|  |  | 720-04-01 |  | x1xx |  | Forward Collision Warning (FCW) braking ON AB (0 selects off, however OFF for Lariat builds?) |
|  |  | 720-09-01 | 3xxx |  |  | Adds evasive steering assist |
|  |  | 720-10-01 | x9xx |  |  | Enables traffic jam assist (TJA) |
|  | IPMA | 706-01-01 |  | 99C9 | A9xx | Updates camera module software (ACC vehicle), updates from Lane Keep Assist retrofit |
|  |  | 706-01-02 | 2076 | 553A | A4 | Tricks IPMA into thinking it’s Lariat |
|  |  | 706-01-03 | xxDx |  |  | Tricks IPMA into thinking it’s Lariat |
|  |  | 706-02-01 |  |  | 9Fxx | 9F pre-selects furthest follow distance |
|  |  | 706-02-02 | FFFD | 55AA | C0xx | No idea what first two hex groups do; C0 selects aid only (no alert, menu optionable) |
|  | CCM | 764-1-01-01 | 4D33 | 4654 | 54 | 4D followed by VIN number of Maverick in hex |
|  |  | 764-1-01-02 | 5738 | 4533 | 38 |  |
|  |  | 764-1-01-03 | 5052 | 4138 | 30 |  |
|  |  | 764-1-01-04 | 3337 | 3800 | 00 |  |
|  |  | 764-1-01-05 | 0000 | 00 |  |  |
| WIRING | CAN HIGH | YELLOW |  |  |  | Connected to IPMA camera, routed down pax A pillar, and joins with RED/BLACK near fuse box. |
|  | CAN LOW | GREEN |  |  |  |  |
|  | RED | HOT / 12v |  |  |  | Connected to fuse #3 on cabin firewall fuse box, switched on |
|  | BLACK | GROUND |  |  |  | Connected below cabin firewall fuse box |
| Door Ajar Chimes Disable | IPC | 720-01-02 | xx0x |  |  |  |
| Triple Ding at Start Up Disable | IPC | 720-01-02 |  | x2-- |  |  |
| Key in Ignition Chime Disable | IPC | EZ IPC |  |  |  | (EZ) Disable "Key Ignition Chime" |
| Double Honk Exiting Running Vehicle Disable | BCM | EZ BodyCM |  |  |  | (EZ) Disable "Double Honk on Leaving Cabin" |
| Driver Restriction Menu Disable (XL,XLT only) | ACM | EZ ACM |  |  |  | (EZ) Disable "Driver Restriction Configuration" |
| Seatbelt Menu and Chimes - Disable All | IPC | 720-13-01 | 0000 | 0000 | 0000 | ALL ZEROS |
|  | IPC | 720-13-02 | 0000 | 0000 |  | ALL ZEROS |
|  | IPC | 720-14-01 | 0000 | 0000 | 0000 | ALL ZEROS |
|  | IPC | 720-14-02 | 0000 | 0000 |  | ALL ZEROS |