FENDER EMBLEM SIDE MARKER AND TURN INDICATOR LIGHTS

Building on the experience of others, I converted my side emblems into side marker and turn indicator lights. I wanted an OEM look. I did not like the commercially available kit as I considered it gimmicky. My build does not require removing the side emblems. Total cost for parts: $52.00. You can also get the functionality of side and turn indicator lights without the lens.

Parts I used:

Purchase automotive-specific electrical tape. Don’t use shiny electrical tape. It will not hold up under the heat in the engine compartment.

Gardener-Bender #15-104,  16 - 14 AWG #8 - 10 Stud Size Ring Terminals in Blue (15-Pack) or equivalent.  Home Depot $3.63

Custom amber lenses. Ordered on Etsy from GadCAD 3D - $24.00

# TRUE MODS 2pc 3/4" Inch Round Amber Trailer 12V LED Marker Light [3 Wire/DRL & Turn Signal] [DOT FMVSS 108] [SAE P2PC] [Semi-Spherical Output] IP67 Waterproof]. Set of two: Amazon $8.99

# 20awg 0.5mm² Electrical Wire 3 Core Extension Cable 20 feet UL2464 DC 5V/12V/24V/300V 20 Gauge Oxygen.  Or equivalent. Amazon $9.59

3M Auto Exterior Attachment Tape, 38582, 1/4 in x 5 ft 3M Auto Exterior Attachment Tape, 38582, 1/4 in x 5 ft. Amazon $4.85

Installation:

I installed these on my Lariat hybrid. I have not had any recalls done. There is a sound cushion in the fender behind the emblem. XL’s and XLT’s don’t have it. The Lariat has LED turn indicator lights. XL’s and XLT’s don’t. On my Lariat there is no problem with hyper flashing. However test as you go, there will be hyper flashing when you disconnect the turn signal connector to test the side light for flashing.

Notes: 1. No need to remove the side emblem. 2. You are working with very thin wires – 20 & 22 gauges. Strip and cut carefully.  3. Push the sound cushion out of the way before drilling. 4. Bench test the LED’s before installation. Use an ordinary 9v. battery. Test both the marker and turn light functions. Note that the LED’s are polarity specific. The common [ground] is negative. 5. Each side is wired with its own ground wire. 6. I am not a fan of the tap style connectors so I soldered and taped all my electrical connections.

Test fit the amber lens to understand how it seats in the emblem. Make a template of the lens out of thick paper. Note that the backside of the lens has a round indentation. Mark the center of the round indentation on the template. This is where the light goes and where you will drill a pilot hole. Flip the template over for the other side.

Drill a pilot hole. Check to insure it is centered on the circle in the lens. Use a sharp ¾” paddle bit aka spade bit or wood boring bit, to drill the hole. The grommet that holds the light will go into the hole.

.

Cut the 20’ piece of 3 wire cable into two 10’ sections. You will need the entire 10’ for each side.

I soldered and taped the wires of the LED light to the wires. If you use tap connectors, be sure they all will go through the ¾” hole. Test the wiring to the LED using a 9v. battery before you poke it through the hole or seat the light into the grommet. Test to see that the lens fits and the LED is centered in the recess in the lens.

Snake the wiring forward. Locate the 8mm ground nut on the frame and be certain your wire reaches. Carefully slice open the insulation on the cable and identify the ground wire. I used the yellow wire.

Cut the ground wire, tin it with solder and crimp on the ground ring. Fasten the ground connector with the 8mm nut. The ground wire is not needed as you proceed forward.

The most difficult part was removing the side market on the driver’s side. The Lariat has all manner of obstructions so removing the fender liner did not allow my hand to reach the market light. Neither did removing the little spoiler. I used an old credit card to pry against and a screw driver to force the assembly forward and out. There are YouTube videos on how to remove them. The side marker on the passenger side can be accessed by removing the fender liner and applying forward pressure while pushing out on the assembly. The factory bulbs are halogen so clean them with rubbing alcohol before reinstalling.

The side marker has two wires. The thicker, black is the ground. The other is the positive. Fish the 3 wire harness cable down, allowing slack to work. Using your preferred method, cut and splice in the proper color wire from the cable for the side marker. Refer to the instructions that came with the LED lights to determine which color wire is for the side marker function.

Route the wiring harness up to the turn signal wiring plug of the headlight assembly. At this point the ground and the side marker wires are inactive as they have been cut. That leaves you with one wire to splice into the turn signal wiring plug. The thinner wire is the one to use. Test for flashing before cutting by poking the wire carefully into the female end of the wiring plug.  You can use your emergency flasher to determine this or you can start your truck and use the turn signal.

I saved applying the 3M tape to the lenses until the very end when I had wired and tested both sides. Clean both the black truck emblem and the surface of the lens with rubbing alcohol before applying the two sided tape.