Ford Maverick Lariat Lux (Hybrid)

VIN: XXXXXXXXXXXXXXXXX

Manufacture Date:	4/22

| Tire and Loading Information |  |  |  |
| --- | --- | --- | --- |
| Seating Capacity | Total: 5 | Front: 2 | Rear: 3 |
| Tire | Size | Cold Tire Pressure |  |
| Front | 225/60R18 100H | 240 KPA, 35 PSI |  |
| Rear | 225/60R18 100H | 240 KPA, 35 PSI |  |
| Spare | 215/60R17 100H | 270 KPA, 39 PSI |  |

| Engine Specifications |  |
| --- | --- |
| Compression Ratio | 13.0:1 |
| Displacement | 152 in3 (2,490 cm3) |
| Firing Order | 1-3-4-2 |
| Ignition System | Coil on Plug |
| Spark Plug Gap | 0.049-0.053 in (1.25-1.35mm) |

| Motorcraft Parts - 2.5L, Hybrid Electric Vehicle |  |
| --- | --- |
| Component | Motorcraft Part Number |
| Air Filter Element | FA-1948 |
| Oil Filter | FL-910-S |
| Battery | BXT-99RT4-A |
| Spark Plug | SP-530 |
| Cabin Air Filter | FP-89 |
| Windshield Wiper Blade | WW-2249 (driver side), WW-2054 (passenger side) |

| Fuel Type | 87 Octane or Greater |
| --- | --- |
| Fuel Capacity | 13.8 / 14.3 |
| Oil Type |  |
| Oil Capacity | 5.7 Quarts |
| Coolant Type |  |
| Coolant Capacity | 11.6 quarts (gas engine)/4.7 quarts (electric) |

Vehicle Code Name:  P758

Vehicle Part Name: VNZ6Z-XXXXXXXX-X

Brakeover Angle . 16.6 Degrees

Paint Code: 2FU/A4D (White)

WHEELS

The 2022 Maverick's factory wheels specs are:

Size: 17x7" or 18x7"
Width: 7" width
Offset: +37.5 mm
Bolt Pattern: 5x108
Center Bore: 63.4
Thread Size: M14 x 1.5 lug nuts
Note: these are the same specs as the Bronco Sport wheels

Standard Wheels:
XL: 17" Steel Wheel Painted Sparkle Silver (27.2 Lbs - source)  17” Continental tires = 24lbs

XLT: 17" Painted Aluminum Wheel

Lariat: 18" Bright Aluminum Wheel

Optional Wheels:
17" Painted Aluminum Wheel [XL & XLT]

17" Unique Aluminum Wheel [FX4 Off Road Package for XLT & Lariat]

18" Black Painted Wheel [Lariat]

TIRES

The 2022 Maverick's tire brands and sizes are:

Continental Procontact TX - [Standard on XL & XLT]
225/65R17 A/S [standard]
225/65/R17 A/T or 235/65R17 A/T [optional]

Michelin Primacy All Season A/S - [Standard on Lariat]
225/60R18 A/S [Optional on FX4 Off Road Package]